ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL TWO | PO: PROPULSION |
| --- | --- |

PO/EO:	416.01

ENABLING OBJECTIVE:	Identify types of engines.

REFERENCE(S):	A.	A-CR-CCP-267/PH-001 Level Two Course Training Plan

Chapter 4, Pages 109-110.

B.	A-CR-CCP-267/PT-001 Level Two Handbook

Chapter 9, Articles 1-8, Pages 1-2.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 49-51 and 81-83.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP Slides

C.	Model Aircraft (if available)

LEARNING AID(S):		A.	A-CR-CCP-267/PT-001 Level Two Handbook

TEACHING POINT(S):	A.	Internal Combustion Engine

B.	Jet Engine

C.	Turboprop Engine

D.	Examples

METHOD OF INSTRUCTION:	Lecture

TIME:		1 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL TWO | PO: PROPULSION |
| --- | --- |

PO/EO:	416.01

ENABLING OBJECTIVE:	Identify types of engines.

REVIEW

PO/EO:	N/A

ENABLING OBJECTIVE:		N/A

INTRODUCTION

WHAT:	In this lesson you will learn about different types of aircraft engine.

WHY:	All aircraft have to have some method of thrust in order to fly.  An understanding of aircraft engines will provide some basic information on how an aircraft engine provides that thrust.

WHERE:	You can apply this knowledge when visiting an aerodrome for a facilities tour, at an air-show, during glider familiarization, or even while attending summer training.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	ENGINE TYPES

5

MINs

1.	General:

Do not

waste time

describing

each engine type in great detail.  Just identify that they operate differently.

a.	Most aircraft have an engine that provides the thrust needed in 			order for it to fly.  Present day aircraft engines are modern				masterpieces of precision workmanship and quality material and 			represent the achievement of years of research and engineering 			experience in	 their design.

b.	Many aircraft have only one engine while others have as many as 			eight.  Engines also come in a variety of sizes and can be one of 			many different types.  The main types of engine:

(i)	Internal combustion engine;

(ii)	The jet engine; and

(iii)	The turboprop engine

2.	Internal Combustion Engine:

a.	Internal combustion engines power most of the small aircraft you

see at the local airport. These engines are designed to turn

propellers in front of, or sometimes behind, a plane. Some

examples are the Cessna 172 and the Piper PA-140 Cherokee.

These are both single engine airplanes that are often used for 			training.

OHP # 1

b.	Internal combustion engines have cylinders with pistons inside 			them.  When the engine is running the cylinders fill with a mixture of 		gasoline and air which enters through the intake valve. The mixture

is compressed by the piston and ignited by a spark plug. The

mixture inside the cylinder quickly burns and forces the piston back 			down the cylinder. The burnt gases are then expelled through the 			exhaust valve and the cycle repeats itself.

c.	This reaction happens very quickly - thousands of times per minute. 		The piston is said to be reciprocating as it moves up and down 			inside the cylinder. The movement of the pistons provides the 			power to turn the aircraft’s propellers.

d.	Propellers are not flat but are curved airfoils. As the propeller

blades strike the air they develop thrust in the same way a wing 			develops lift.

CONFIRMATION STAGE 1

1.	What does reciprocating refer to?

Answer:	The up and down movement of the piston in an internal				combustion engine.

2.	How does an internal combustion provide thrust?

Answer:	The movement of the pistons inside an internal combustion 				engine provides the power to turn the aircraft’s propellers,				which in turn provides the aircraft with thrust.

STAGE 2	ENGINE TYPES (continued)

15

MINs

1.	Jet Engine:

a.	Some types of aircraft are powered by jet engines.  Modern military 			aircraft like the F-14, the F-15 and the CF-18 use types of jet				engines.  Many airliners such as the Boeing 747 also use types of 			jet engines.

b.	In a jet engine, incoming air is compressed and rammed into a			combustion chamber by a compressor, which is driven by a turbine 			wheel. Hot exhaust gases drive this turbine wheel, called the

OHP # 2

compressor turbine.

c.	In the combustion chamber, fuel is added to the compressed air			and burned. This produces a hot, high-speed gas. It then flows			through the turbine wheel, which uses just enough energy to power			the compressor. The rest of the energy provides thrust in the form 			of hot gases being blown out the back in a steady stream.

d.	Because the compressor is run by the same engine it powers, the 			jet has to be started with a starter motor.

OHP # 3

2.	Turboprop Engine:

OHP # 1

The internal combustion engine and jet engine are really the two basic

aircraft engine types. However, in addition to jet propulsion, a jet engine

can be used to drive a propeller. In this type of engine, a jet is used to

power a turbine wheel called the power turbine. This turbine drives the

propeller. This style of engine is referred to as a turboprop engine. This

type of engine is ideal for heavy load carriers like the Hercules. It is also

used on the Dash 8 and the Aurora.

CONFIRMATION STAGE 2

1.	How does a jet engine provide thrust?

Answer:	A jet engine provides thrust by blowing hot gases out the				back of the jet in a steady stream.

2.	Name three aircraft that use the turbo prop engine.

Answer:	The Dash 8, Hercules and the Aurora.

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - 	Each cadet will have to identify the three different types of engine.

1.	Test knowledge with OHP #4.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you have learned about different types of aircraft engine.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You can apply this knowledge when visiting an aerodrome for a facilities		tour, at an air-show, during glider familiarization, even while attending 	summer training.

C.	Your next lesson is PO/EO 416.02. In this class you will learn about the components of an internal combustion engine.