ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL TWO | PO: PROPULSION |
| --- | --- |

PO/EO:	416.03

ENABLING OBJECTIVE:	Explain the four-stroke cycle.

REFERENCE(S):	A.	A-CR-CCP-267/PH-001 Level Two Course Training Plan

Chapter 4, Pages 114-115.

B.	A-CR-CCP-267/PT-001 Level Two Handbook

Chapter 9, Articles 16-21, Pages 4 and 6.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Page 51.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP Slides

C.	Model Aircraft (if available)

LEARNING AID(S):		A.	A-CR-CCP-267/PT-001 Level Two Handbook

TEACHING POINT(S):	A.	Intake stroke

B.	Compression stroke

C.	Combustion (power) stroke

D.	Exhaust stroke

METHOD OF INSTRUCTION:	Lecture

TIME:		1 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL TWO | PO: PROPULSION |
| --- | --- |

PO/EO:	416.03

ENABLING OBJECTIVE:	Explain the four-stroke cycle.

REVIEW

PO/EO:	416.02

ENABLING OBJECTIVE:		Identify the components of an internal combustion engine.

1.	Review weaknesses of the last EOs performance check; and/or

2.	Ask the following review questions:

a.	Name the main components of an internal combustion engine.

Answer:	Cylinder, intake valve, exhaust valve, spark plugs, piston, connecting

rod and crankshaft.

b.	What are the three types of cylinder configuration?

Answer:	Horizontally opposed, radial and in-line.

INTRODUCTION

WHAT:	In this lesson you will learn about the four-stroke cycle of an internal combustion engine.

WHY:	All aircraft have to have some method of thrust in order to fly.  An understanding of aircraft engines will provide some basic information on how an aircraft engine provides that thrust.  The four-stroke cycle is the basic principle behind the power production of an aircraft engine.

WHERE:	You can apply this knowledge when visiting an aerodrome for a facilities tour, at an air-show, during glider familiarization, or even while attending summer training.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	THE FOUR-STROKE CYCLE

5

MINs

1.	General:

a.	The parts of an engine work together in a cycle to turn the aircraft’s			propeller. In most aircraft engines this cycle has four distinct stages			called strokes. The piston travels four strokes (two up and two

down) to complete one cycle.

b.	During this operation, the crankshaft revolves through two complete 		revolutions. Each of the four strokes is identified by describing what

the engine parts are doing in each stage.

2.	The Intake Stroke:

OHP # 1

a.	During the first stroke the intake valve opens to let the gasoline and			air mixture into the cylinder and the piston moves down to draw the 			mixture into the cylinder.

b.	The exhaust valve is closed during this stage.

3.	The Compression Stroke:

OHP # 2

In the second stroke both valves are closed while the piston moves 	up to

compress the mixture

CONFIRMATION STAGE 1

1.	What are the first two strokes of the four-stoke cycle?

Answer:	The intake stroke and compression stroke.

2.	What happens in these stages?

Answer:	a. 	During the intake stroke the intake valve opens to let

the gasoline and air mixture into the cylinder and the

piston moves down to draw the mixture into the

cylinder. The exhaust valve is closed during this

stage.

b.	During the compression stroke both valves are closed

while the piston moves up to compress the mixture

STAGE 2	THE FOUR-STROKE CYCLE

OHP # 3

15

MINs

1.	The Combustion (Power) Stroke:

In the third stroke both valves remain closed while the spark plug ignites

the gas, which burns, expands and forces the piston down again.

2.	The Exhaust Stroke:

OHP # 4

a.	In the fourth stroke the intake valve is closed while the exhaust 			valve opens to let the burnt gases out.  The piston moves up 			again to forces the gases out.

b.	After the exhaust stroke, the whole process repeats itself causing 			the crankshaft to turn the propeller on the aircraft.

CONFIRMATION STAGE 2

1.	What are the last two strokes of the four-stoke cycle?

Answer:	The combustion (power) stroke and exhaust stroke.

2.	What happens in these stages?

Answer:	a.	During the combustion stroke both valves remain

closed	 while the spark plug ignites the gas, which

burns, expands and forces the piston down again.

b.	During the exhaust stroke the intake valve is closed

while the exhaust valve opens to let the burnt gases

out. The piston moves up again to forces the gases

out.

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - 	Each cadet will have to explain the four-stroke cycle of an internal

combustion	engine.

1.	Test knowledge with OHP #5. It may be either projected or photocopied and distributed.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you have learned about the four-stroke cycle of an internal 	combustion engine.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You can apply this knowledge when visiting an aerodrome for a facilities		tour, at an air-show, during glider familiarization, or even while attending 	summer training.

C.	Your next lesson is PO/EO 416.04. In this class you will learn about the functions of oil in an engine.