ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL THREE | PO: PROPULSION |
| --- | --- |

PO/EO:	416.02

ENABLING OBJECTIVE:	Identify engine instruments.

REFERENCE(S):	A.	A-CR-CCP-268/PH-001 Level Three Course Training Plan

Chapter 4, Pages 103-106.

B.	A-CR-CCP-268/PT-001 Level Three Handbook

Chapter 9, Section 7, Articles 41-42, Pages 13-18.

C.	A-CR-CCP-263/PT-001 From the Ground Up,

Pages 72-74.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP Slides

C.	Aircraft Instruments (if available)

LEARNING AID(S):		A.	A-CR-CCP-268/PT-001 Level Three Handbook

TEACHING POINT(S):	A.	The oil pressure gauge

B.	The oil temperature

C.	The cylinder head temperature gauge

D.	The carburetor air temperature gauge

E.	The outside air temperature gauge

F.	The tachometer

G.	The manifold pressure gauge

METHOD OF INSTRUCTION:	Lecture

TIME:		1 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL THREE | PO: PROPULSION |
| --- | --- |

PO/EO:	416.02

ENABLING OBJECTIVE:	Identify engine instruments.

REVIEW

PO/EO:	416.01

ENABLING OBJECTIVE:		Explain Propellers

1.	Review weaknesses of the last EOs performance check; and/or

2.	Ask the following review questions:

a.	How does a propeller work?

Answer:	A propeller blade is an airfoil section, similar to the airfoil section of a wing.  When a propeller starts rotating, it creates the same forces that are created when air flows over an aircraft wing: lift and drag.  However, in the case of a propeller, these forces are designated as thrust and torque.

b.	What is pitch?

Answer:	The pitch is the forward distance traveled by the propeller in one						 revolution.

INTRODUCTION

WHAT:	In this lesson you will learn about aircraft engine instruments.

WHY:	Engine instruments indicate vital information to the pilot and therefore are an important safety factor.

WHERE:	You can apply this knowledge when visiting an aerodrome for a facilities tour, at an air-show, during glider familiarization, or even while attending  summer training.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	ENGINE INSTRUMENTS

5

MINs

1.	General:

a.	An engine, whether it be that of a car or an aircraft, is a grouping of 			complex mechanical parts that are interrelated and from which we 			demand great power and efficiency. Sometimes, the demands 			made on each metal part cause overheating or an increase in 			pressure, indicating poor functioning.

b.	Pilots have access to a series of dials that provide them with 				essential information regarding the engines status. Therefore, they

can monitor the engine so that they can quickly identify, assess and 		deal with any engine problems that occur

2.	The Oil Pressure Gauge:

OHP #1

a.	The oil pressure gauge is one of the principal engine instruments.  			It indicates the oil pressure supplied by the oil pump to lubricate the 		engine.

b.	High oil pressure will force oil into the combustion chamber. There 			it will burn, causing a smoky exhaust, and badly carbonate piston 			heads, valve heads and cylinder heads.

c.	If the pressure were permitted to drop low enough, there would be 			no film of oil between the working surfaces of the engine. Metal 			would be rubbing on metal causing extensive damage and possibly 			engine seizure.

3.	The Oil Temperature Gauge:

OHP #1

a.	The oil temperature gauge gives a reading of the temperature of 			the oil in degrees Fahrenheit or Celsius. The relationship between 			the oil temperature and oil pressure is critical.

b.	Extremely low temperature is undesirable. Cold oil does not 				circulate freely and may cause scoring of the engine parts.  Low 			temperature would be accompanied by a corresponding rise in 			pressure.  It should always	be remembered that oil, in addition to 			lubrication, acts as a coolant.

c.	Extremely high temperature is also undesirable. High pressure is

usually accompanied by a drop in pressure. However, even when 			the pressure shows no marked change, increasing oil temperature 			is a warning of excessive friction or overload in the engine.

4.	The Cylinder Head Temperature Gauge:

OHP #2

a.	The cylinder head temperature gauge records the temperature of 			one (or more) of the cylinder heads.

b.	This instrument gives a reasonably good indication of the				effectiveness of the engine cooling system.  High temperature			decreases the strength of metals and can result in detonation, pre-			ignition, and eventual engine failure.

OHP #2

5.	The Carburetor Air Temperature Gauge:

a.	The carburetor air temperature gauge may be installed to record 			the mixture temperature or to record the intake air temperature.

b.	The carburetor air temperature gauge enables the pilot to maintain 			a temperature that will assure maximum operating efficiency and 			warn of icing conditions in the carburetor that may lead to engine 			failure.

CONFIRMATION STAGE 1

1.	Confirm the cadets’ knowledge with OHPs from Stage 1.

STAGE 2 	ENGINE INSTRUMENTS

1.	The Outside Air Temperature Gauge:

a.	The outside air temperature gauge records the ambient air 				temperature, that is, the temperature of the free air surrounding the 			aircraft.

b.	The temperature recorded by the gauge enables the pilot to select			the proper manifold pressure, to calculate the true airspeed and 			altitude and warn of conditions that may cause ice formation.

2.	The Tachometer:

OHP #3

a.	The tachometer is an instrument that shows the speed at which the 			engine crankshaft is turning in hundreds of revolutions per minute 			or rpm. The rpm is directly proportional to the power output of the 			engine.

b.	On an aircraft fitted with a fixed pitch propeller, the tachometer is 			the only instrument that displays information about engine power 			output.

3.	The Manifold Pressure Gauge:

OHP #3

a.	The manifold pressure gauge indicates, in inches of mercury, the 			pressure of the fuel/air mixture in the engine intake manifold at a 			point between the carburetor and the cylinders.

b.	On aircraft fitted with a controllable pitch or constant speed pitch 			propeller, the tachometer and manifold pressure gauge are the two 			instruments that display information about engine power output.

CONFIRMATION STAGE 2

1.	Confirm the cadets’ knowledge with OHPs from Stage 2.

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - Each cadet will have to identify each engine instrument.

1.	Test knowledge with OHPs. They may be either projected or photocopied and distributed.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you have learned about aircraft engine instruments.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You can apply this knowledge when visiting an aerodrome for a facilities tour, at an air-show, during glider familiarization, or even while attending summer training.

C.	Your next lesson is PO/EO 416.03. In this class you will learn about an aircraft’s fuel system.