ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.03a

ENABLING OBJECTIVE:	Identify navigation terms and units of distance and speed.

REFERENCE(S):	A.	A-CR-CCP-269/PH-001 Level Four Course Training Plan

Chapter 4, Pages 85-89.

B.	A-CR-CCP-269/PT-001 Level Four Handbook

Chapter 9, Section 5, Pages 1-5.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 181 and 184.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	OHP

B.	OHP Slides

LEARNING AID(S):		A.	A-CR-CCP-269/PT-001 Level Four Handbook

TEACHING POINT(S):	A.	Navigation Terms

METHOD OF INSTRUCTION:	Lecture

TIME:		1 of 2 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.03a

ENABLING OBJECTIVE:	Identify navigation terms and units of distance and speed.

REVIEW

PO/EO:	417.02b

ENABLING OBJECTIVE:		Identify navigation charts.

1.	Review weaknesses of the last EOs performance check; and/or

2.	Ask the following review question:

- a.	Name the three types of map scale.

- Answer:	Representative fraction, statement of words and graduated scale.

INTRODUCTION

WHAT:	In this lesson you will learn about important navigation terms

WHY:	It is necessary to understand navigation terms in order to plot a flight plan.

WHERE:	You will have an opportunity to use this knowledge during the last Navigation class, where you will plot a flight plan.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	NAVIGATION TERMS

5

MINs

1.	Navigation Terms:

a.	Indicated Airspeed:

OHP #1

This is the airplane's speed as indicated by the 	airspeed indicator.

b.	True Airspeed:

This is the actual speed of the airplane through the air	mass.  It is

indicated airspeed corrected for the airspeed indicator	error due to

density and temperature.

c.	Groundspeed:

This is the speed of the airplane relative to the ground. An airplane

is affected by wind. If there is no wind at all, true airspeed and

groundspeed will be the same. If, however, an airplane is flying in

an air mass that is moving in the same direction, the airplane will

have a tailwind that will help its progress over the ground, with the

result that its groundspeed will be in excess of the true airspeed.

Conversely, a headwind will impede the progress of he airplane

over the ground, with the result that the groundspeed will be slower

than the true airspeed.

d.	Magnetic Meridian:

OHP #2

This is the direction in which a compass needle will lie when

influenced only by the Earth's magnetic field. In actual practice,

magnetic meridians are not shown on maps but are found by

adding or subtracting the variation at any particular place to or from

the true meridian. (Variation is indicated on maps by isogonic lines,

which are lines joining all places of equal variation.)

e.	Compass North:

This is the direction in which a particular compass needle will lie

when influenced by both the Earth's magnetic field and local

magnetic influences (deviation) in the airplane. The actual reading

on a compass at any time is the angle between compass north and

the direction in which the airplane is heading.

f.	Azimuth:

OHP # 3

This refers to the direction to or from any point, measured as an

angle clockwise from a meridian. It is the same as a bearing. The

azimuth or bearing may be true, magnetic or compass.

g.	Wind

This is air in motion, especially a mass of air having a common

direction or motion. Wind moves horizontally. (A movement of air 			vertically is called a current).

h.	Drift:

A wind blowing from either the starboard or portside of an airplane 			will cause the airplane to Drift away from its intended track. In order

to maintain the intended track, it is necessary to turn the airplane

slightly into wind to compensate for the force acting laterally upon it.

Drift, or drift angle, is the angle between the heading being flown

and the track made good over the ground. In other words, it is the

angle at which the pilot heads the airplane across the track to keep

the wind from blowing the plane off the track. It is expressed in

degrees either port of starboard.

CONFIRMATION STAGE 1

1.	What is groundspeed?

Answer:	Groundspeed is the speed of the airplane relative to the				ground.

2.	What is an azimuth?

Answer:	Azimuth refers to the direction to or from any point, 					measured as an angle clockwise from a	meridian.  It is the 				same as a bearing.  The azimuth or bearing may be true, 				magnetic or compass.

STAGE 2 	NAVIGATION TERMS

17

MINs

1.	Navigation Terms:

OHP # 4

a.	Heading:

The Heading of an airplane is the angle between the longitudinal 			axis of the airplane at any moment and a meridian. In other words, 			it is the direction the nose of the airplane is pointing, measured 			from an imaginary line running north and south. If the heading is 			measured from a true meridian, it is referred to as a True Heading.  			If the heading is measured from a magnetic meridian, it is called a 			Magnetic Heading. If it is measured from the direction of a 				compass needle, it is referred to as a compass heading. The angle

is measured clockwise through 360º.  Note that this is different than

an azimuth or bearing. A bearing from an aircraft to a particular			location would not necessarily be the same as the direction the 			plane was heading.

b.	Track (intended):

This is the direction an airplane intends to travel over the ground.

The intended track may be represented by a straight line drawn on

a map. Its direction is the angle between this line and a meridian,

measured clockwise through 3600. As in the case of headings,

tracks are named true, magnetic or compass with reference to the

meridian from which they are measured.

c.	Track Made Good:

OHP # 5

This is the actual path traveled by the airplane over the ground.

Like the intended track, it may be represented by a line drawn on a

map and (provided it is a reasonably straight line), its direction

measured from a true or magnetic meridian or compass north.

d.	Track Error:

This is the angle between the track and the track made good,

measured in degrees either left or right of the required track.

OHP # 6

e.	Opening Angle

This is the angle between the required track and the track made

good.

f.	Closing Angle:

This is the angle between the old required track and the new

required track necessary to arrive at the destination.

CONFIRMATION STAGE 2

1.	What is heading?

Answer:	The Heading of an airplane is the angle between the					longitudinal axis of the airplane at any moment and a 					meridian. In other words, it is the direction the nose of the 				airplane is pointing. It can be measured from a true					meridian, a magnetic meridian or from a compass needle.

2.	What is track (intended)?

Answer:	The Track (intended) is the direction an airplane intends to 				travel over the ground. The intended track may be 					represented by a straight line drawn on a map. Its direction 				is the angle between this line and a meridian, measured 				clockwise through 3600. As in the case of headings, tracks 				are named true, magnetic or compass.

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - 	Each cadet will have to identify navigation terms.

1.	Test knowledge with OHPs.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you have learned about navigation terms.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You will have an opportunity to use this knowledge during the last 				Navigation class, where you will plot a flight plan.

C.	Your next lesson is PO/EO 417.03b. In this class you will learn about units of distance and speed.