ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.01b

ENABLING OBJECTIVE:	Review Level Three Navigation.

REFERENCE(S):	A.	A-CR-CCP-269/PH-001 Level Four Course Training Plan

Chapter 4, Pages 82.

B.	A-CR-CCP-268/PT-001 Level Three Handbook

Chapter 10.

C.	A-CR-CCP-263/PT-001 From the Ground Up

Pages 164-165 and 179-180.

SUPPLEMENTARY REF(S):	N/A

TRAINING AID(S):		A.	VFR Navigation Charts (VNC)

B.	Protractor or Plotter

LEARNING AID(S):		A.	A-CR-CCP-268/PT-001 Level Three Handbook

B.	VFR Navigation Charts (VNC)

C.	Protractor or Plotter

# D.	Rulers, Pencils and Erasers

TEACHING POINT(S):	A.	Determine the position of a plane

B.	Determine the direction of a plane

METHOD OF INSTRUCTION:	Review

TIME:		2 of 2 x 35-minutes

ROYAL CANADIAN AIR CADETS

MASTER LESSON PLAN

| PROFICIENCY LEVEL FOUR | PO: NAVIGATION |
| --- | --- |

PO/EO:	417.01b

ENABLING OBJECTIVE:	Review Level Three Navigation.

REVIEW

PO/EO:	N/A

ENABLING OBJECTIVE:		N/A

INTRODUCTION

WHAT:	In this lesson you will review Level Three Navigation.

WHY:	Knowledge of technical navigation procedures, as they apply to pilots, will help cadets understand the importance of navigation principals in plotting flights. Cadets must understand Level 3 Navigation thoroughly before they can move on to Level Four Navigation.

WHERE:	You will have an opportunity to use this knowledge during the last navigation class, where you will plot a flight plan.

| TIME | BODY | NOTES |
| --- | --- | --- |

STAGE 1	LEVEL THREE NAVIGATION REVIEW

5

MINs

1.	Review Questions:

a.	How do you determine the position of a plane?

Answer:	(i)	The position of cities, airports and aircraft can

may be designated by their geographical co-

ordinates, that is, the intersection of the lines of

latitude and longitude marking their position on

a map.

(ii)	When a position is being identified by

geographical co-ordinates, the latitude co-

ordinate is written first and is followed by the

longitude coordinate. The latitude and

longitude of Winnipeg, Manitoba will be written

as 49054’N 97014’W, meaning 49 degrees 54

minutes north of the	equator and 97 degrees

14 minutes west of the prime meridian.

(iii)	Latitude and longitude are indicated differently

on different types of  navigation charts. VFR

(Visual Flight Rules) Navigation Charts (VNC)

will be described here. These charts are used

for visual navigation at lower altitudes and

airspeeds.

(iv)	The top of a VNC is oriented towards true

north. Thus, parallels of latitude run across the

VNC from side to side and meridians of

longitude run from top to bottom.

Get the

cadets to determine several positions

on a VNC.

(v)	On a VNC, the meridians of longitude and the

parallels of latitude are numbered at the edge

of the chart. The parallels and meridians are

also conveniently numbered within the body of

the chart. Parallels and meridians, at 30’

intervals, are divided into 1-minute divisions.

b.	How do you determine the direction of a plane?

Answer:	(i)	When planning a flight, pilots draw a line

between the departure point and the expected

arrival point. They then calculate the angle this

line makes with a meridian of longitude. The

angle is always measured clockwise from the

15

MINs

particular chosen meridian using a compass

rose. It is best to measure this angle using a

protractor or plotter.

(ii)	A protractor, being square, can be used both

for determining direction and as a straightedge.

The instrument has a compass rose graduated

in 360º marked around the outer edges. It is

transparent so that, when placed on a map, the

map is visible through it.

(iii)	Place the protractor on the map with the hole in

the centre lying on the track at a point where

the north-south line on the protractor lies along

a meridian. If this is not convenient, the lines

may be lined up parallel with the nearest

meridian. The direction is read off where the

track line cuts the edge of the protractor. Make

sure you read the protractor on the side closest

to the destination point.

(iv)	A plotter combines a protractor and a

straightedge in one device. It also incorporates

a mileage scale for both 1:500,000 and

1:1,000,000 charts. The plotter is made of clear

plastic so that details of the chart can be seen

through it.

(v)	With the straightedge, the pilot can draw a

track from the airport of departure to the

planned destination. The direction of a track is

determined by using the protractor portion of

the plotter. It is numbered from 000º to 180º on

the outside scale and from 190º to 360º on the

inside scale. The outside scale is for easterly

tracks and the inside scale for westerly tracks.

(vi)	To use the plotter, place the hole in the centre

of the plotter over an intersection of the track

line and one of the longitude lines on the chart.

A point somewhere near the mid point of the

track is best chosen to obtain greater accuracy.

Get the

cadets to determine several

directions

on a VNC

with either

a protractor

or a plotter.

(vii)	Place a pencil point through the hole and rotate

the plotter until the top edge of the straightedge

is aligned with the track line.  Read the

direction of the track in degrees true where the

longitude line of the chart intersects the scales.

(Use the outer scale for easterly tracks and the

inner scale for westerly tracks)

| 28 MINs | PERFORMANCE CHECK |
| --- | --- |

Test Details - 	Each cadet will have to demonstrate an adequate knowledge of Level Three

Navigation.

1.	Test knowledge by getting the cadets to practice the skills being reviewed.

| 33 MINs | CONCLUSION |
| --- | --- |

SUMMARY:	A.	In this lesson you reviewed Level Three Navigation.

RE-MOTIVATION:	A.	Comment on student performance. (Identify strengths and points that

require improvement)

B.	You will have an opportunity to use this knowledge during the last 				Navigation class, where you will plot a flight plan.

C.	Your next lesson is PO/EO 417.02a.  In this class you will learn about navigation charts.